# Network Architecture and Protocol Lab

Booklet 7

Submitted by: Jamil Ahmed

Neptun Code: JZT75Y

# 4.1

A) What modes do we have to configure on each network adapter?

- Internal Network. Used between VMs (net1, net2, net4)

- NAT. Used for Internet access (network 3)

B) What command the IP addresses can be configured with?

Ifconfig <interface> inet6 add <IPv6-address>/<prefix>

# 4.2

A) What is the meaning of the routing table?

The routing table is a data structure maintained by the operating system that determines how packets are forwarded to their destination. It contains entries that specify destination networks, the next-hop address (gateway), and the outgoing interface. When a packet is sent, the system consults this table to decide whether the destination is directly reachable or must be forwarded through a router.

B) What command the access of a direct network can be configured with?

ip -6 route add <network>/<prefix> dev <interface>

C) What command the access of a remote network can be configured with?

ip -6 route add <network>/<prefix> via <gateway-address>

# 4.3

A) How can we find the number of routers between two nodes? Determine the command.

traceroute6 <destination IPv6>

B) What does RTT (Round-Trip Time) value prove?

RTT (Round-Trip Time) represents the time it takes for a packet to travel from the source to the destination and back again. It indicates network latency and helps evaluate the performance and responsiveness of the network.

4.4

A) How many 16-bit fields does Ipv6 address in a preferred form contain?

An IPv6 address in its preferred (full) form contains 8 fields, each consisting of 16 bits, making a total of 128 bits. These fields are written in hexadecimal and separated by colons.

B) Which compression rules did you apply to shorten the Ipv6 address?

Two compression rules were applied: first, leading zeros in each 16-bit field were removed, and second, consecutive fields containing only zeros were replaced with a double colon (::). The double colon can be used only once in an address to avoid ambiguity.

# 4.5

A) How many IPv6 addresses does the network adapter of your VM have?

A network adapter typically has more than one IPv6 address. At minimum, it has a link-local address (FE80::/10), which is automatically generated, and a global unicast address, which is manually assigned or obtained from the network. Therefore, each adapter usually has at least two IPv6 addresses.

B) In what way were the interface ID parts of the Ipv6 addresses of the network adapter produced? Justify your answer.

The interface ID part of the IPv6 address is often generated automatically using the EUI-64 method, which is based on the MAC address of the network interface. This method inserts additional bits into the MAC address and modifies it to create a unique 64-bit identifier.

# 4.6

A) Can a routing table include static routes?

Yes, a routing table can include static routes. These are manually configured routes that define specific paths to reach certain networks.

B) What is the difference between the access of a remote network and that of a direct network?

The main difference is that a direct network is one that is physically connected to the device, so packets can be delivered directly through the corresponding interface. In contrast, a remote network is not directly connected and requires forwarding through a router (next-hop). Therefore, direct access uses an interface, while remote access requires a gateway.

# 4.7

- Above which protocol does the traceroute6 command operate by default?

The traceroute6 command operates over the ICMPv6 protocol by default. It sends ICMPv6 Echo Request messages with increasing hop limits to discover the path to the destination.

- What do the three asterisks (***) denote in the output of traceroute command?

The three asterisks (***) in traceroute output indicate that no response was received from that hop within the timeout period.